Pi Zero OLED Configuration

Setup per:

https://www.raspberrypi-spy.co.uk/2018/04/i2c-oled-display-module-with-raspberry-pi/

By way of explanation for some of the comments below, I generally work on a Mac, running SSH to work on the Raspberry Pi. In this process, I generally do any file editing on the Mac (BBEdit) and transfer files between the Mac and Pi as required (via SFTP using Transmit).

Installation notes:

Enable I2C:

sudo raspi-config
	[5 Interfacing Options -> P5 I2C (Enable I2C)]

Ensure that the following are installed:

sudo apt install -y python3-dev

sudo apt install -y python-smbus i2c-tools

sudo apt install -y python3-pil

sudo apt install -y python3-pip

sudo apt install -y python3-setuptools

sudo apt install -y python3-rpi.gpio

Check the I2C address of the display:

i2cdetect -y 1

pi@BB-7:~ $ i2cdetect -y 1

0  1  2  3  4  5  6  7  8  9  a  b  c  d  e  f
00:          -- -- -- -- -- -- -- -- -- -- -- -- -- 
10: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- 
20: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- 
30: -- -- -- -- -- -- -- -- -- -- -- -- 3c -- -- -- 
40: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- 
50: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- 
60: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- 
70: -- -- -- -- -- -- -- --

pi@BB-7:~ $

Install the Python OLED library

sudo apt-get install git
sudo git clone https://github.com/rpsreal/pySX127x

sudo apt install -y git

pi@BB-7:~ $ sudo apt install -y git

Reading package lists... Done
Building dependency tree       
Reading state information... Done
git is already the newest version (1:2.20.1-2).
0 upgraded, 0 newly installed, 0 to remove and 0 not upgraded.

pi@BB-7:~ $

git clone https://github.com/adafruit/Adafruit_Python_SSD1306.git

pi@BB-7:~ $ git clone https://github.com/adafruit/Adafruit_Python_SSD1306.git
Cloning into 'Adafruit_Python_SSD1306'...
remote: Enumerating objects: 137, done.
remote: Total 137 (delta 0), reused 0 (delta 0), pack-reused 137
Receiving objects: 100% (137/137), 43.32 KiB | 372.00 KiB/s, done.
Resolving deltas: 100% (70/70), done.

pi@BB-7:~ $

cd Adafruit_Python_SSD1306

sudo python3 setup.py install

Everything else seems to be OK with the default settings, except for the sync word, which was explicitly set to 0x12 in LoRa.py:

@setter(REG.LORA.SYNC_WORD)
def set_sync_word(self, sync_word=0x12):
    return sync_word

This setting was subsequently moved to the main Python script (LORA_PI_RX.py):

lora.set_sync_word(0x12)

I also had to fiddle around a bit (because I’d never previously worked with Pytho) to print the output as a hex string rather than whatever it was originally (LORA_PI_RX.py):

print(bytes(payload).hex())

This operation was subsequently enhanced considerably by adding a new method (print_packet(self, payload)) to break out the packet content.

To match the current hardware configuration, which is a little different to the original, I had to modify the board_config.py file to specify the new pin configuration:

DIO0 = 25   # RaspPi GPIO 25
DIO1 = 24   # RaspPi GPIO 24
DIO2 = 23   # RaspPi GPIO 23
DIO3 = 27   # RaspPi GPIO 27
RST  = 22   # RaspPi GPIO 22
LED  = 13   # RaspPi GPIO 13

set the low_band parameter to reflect the fact that we’re operating at 917.0MHz:

low_band = False

and remove the second board definition (just to be tidy, because we don’t have a second board).

The second board configuration details were also removed from the lora.py file.

This configuration was chosen for two reasons. First, it clustered the connections together on the Raspberry Pi, making it easier to build a ‘hat’ for the job, and second, to make the configuration compatible with that described by Charles-Henri Hallard (https://github.com/hallard/LoRasPI), which is reportedly supported by the default LMIC setup (which I might get around to testing one day, because, as I understand, the LMIC library is available for all the platforms I am using).